| TECHNICAL SERVICE BULLETIN Whistle Noise At Lower Left Windshield Area - Built On Or Before 28-Apr-2022 | 22-2306 01 August 2022 |
| --- | --- |
| Model: Ford 2022 Bronco; Built on or before 28-Apr-2022. Issue: Some 2022 Bronco vehicles may exhibit a whistle noise below the lower left windshield area at the bottom of the A-pillar. This may be due to a missed body sealer application at the factory. To correct this condition, follow the Service Procedure to reseal the affected body seam. Action: Follow the Service Procedure to correct the condition on vehicles that meet all of the following criteria: • 2022 Bronco • Built on or before 28-Apr-2022 • Whistle noise from the lower left windshield area Parts Service Part Number; Quantity; Description. Obtain Locally; As Needed; 3M® 2 Inch Wide Masking Tape Or Equivalent. Obtain Locally; As Needed; Butyl - 3M® 08612 Windo Weld Round Ribbon Sealer Or Equivalent. As Needed indicates the amount of the part may vary and/or is not a whole number. Parts can be billed out as non-whole numbers, including less than 1. Warranty Status: Eligible under provisions of New Vehicle Limited Warranty (NVLW)/Service Part Warranty (SPW)/Special Service Part (SSP)/Extended Service Plan (ESP) coverage. Limits/policies/prior approvals are not altered by a TSB. NVLW/SPW/SSP/ESP coverage limits are determined by the identified causal part and verified using the OASIS part coverage tool. Labor Times Description; Operation No.; Time. 2022 Bronco: Install Tape, Road Test (Pass) Inspect Gap And Apply Butyl Sealer If Necessary (Do Not Use With Any Other Labor Operations); 222306A; 1.2 Hrs.. Repair/Claim Coding Causal Part:; 5802012. Condition Code:; 68. Service Procedure 1. Tape off the left side of the cowl panel and drive the vehicle above 50 mph (80 Km/h). Did taping off the cowl resolve the whistle noise? (Figure 1) Figure 1 (1). Yes - proceed to Step 2. (2). No - this article does not apply. Refer to Workshop Manual (WSM), Section 100-04. 2. Remove the left A-pillar trim. Refer to WSM, Section 501-05. 3. Visually inspect for a gap in the sheet metal at the bottom of the A-pillar where it joins the rear engine compartment wall. Is a gap present? (Figure 2) Figure 2 (1). Yes - proceed to Step 4. (2). No - this article does not apply. 4. Apply a liberal amount of butyl to fill the gap between the two metal panels and eliminate the air entry. 5. To assemble, reverse the removal procedure. |